1.Input

(1)3DoFs: accelerometers; gyroscopes

(2) 追踪器性能指标

准确度 (Accuracy): The similarity between the measured value and the actual position; the further away from the origin of the reference coordinate system, the lower the accuracy.

抖动 (Jitter): The random noise in the tracker output, even when the tracked object remains stationary.

漂移 (Drift): The phenomenon where error accumulation and increase over time.

延迟 (Latency): The time delay between the moment of action and the system's output of the signal.

(3) Types of 3D Trackers

Mechanical Trackers:(adv) Accurate; small latency; no line-of-sight restriction (disadv) Interfere with the user's movement

Magnetic Trackers: transmitter generate magnetic field; the receiver senses the changes of magnetic field to calculate the position. (AC: Eddy Currents issue) (DC: use Pulse emission mode)

Ultrasonic Trackers: Distance measurement using the flight time of sound waves ,and use it to calculate the position(adv:price low; Not affected by metals)(disadv: line-of-sight restriction; affected by temperature, environmental noise; low update rate)

Optical Trackers: (Outside Looking In: The external camera captures the marks on the user's body)( Inside Looking Out: The sensor is on the user and marked in the environment)(adv: high accurate)(disadv: Occlusion marks on body parts cannot be traced.;ambiguity)

Inertial Trackers(gyroscope: detect angular velocity; accelerometer: detect linear acceleration; magnetometer: detect the intensity of Earth’s magnetic field)(adv)no line-of-sight restriction; small\cheap\fast; low jitter(disadv) use accelerometer, has drift issue

(4) Gesture Devices

Pinch Gloves; 5DT Data Gloves; Didjigloves; CyberGloves

2.Output

(1) 视网膜生理结构

Rods：night vision

Cones：color vision，L/M/S cones 对 red/green/blue 敏感「绿色最敏感」。

(2) FOV

Each eye：约 150° horizontal，135° vertical。

Both eyes combined：约 180° horizontal。

Binocular overlap：约 120° horizontal。

The brain relies on the horizontal offset of images in overlapping areas to determine depth

(3) Depth Cues

Near object cues: convergence[eyes turn in]\ accommodation[lens change shapes]\ Disparity[two different views]\ Motion Parallax[fast vs. slow movement]

Far object cues: occlusion\perspective\	haze\surface texture

(4) Convergence-accommodation conflict

Eyes converge to virtual object depth but accommodate to the fixed display/virtual image plane. This mismatch causes discomfort.

(5)Sound Localization

方位角:ITD (Interaural Time Difference 双耳时间差) & IID (Interaural Intensity Difference，双耳强度差); 低频only by ITD

仰角: Different frequencies are caused by the physical reflection of the pinna

Distance Cues: Loudness/prior knowledge & motion parallax

(6)HRTF: Perform a transform on the relationship between the sound emitted by the sound source and the sound reaching the ear

(7) Haptic Feedback

Tactile feedback: vibration, texture, pressure, temperature

Force feedback: weight, resistance, collision

(8) CAVE 系统和多投影穹顶 (Domes) 为了产生无缝的大图像，必须使用哪两种关键处理技术？

Blending (边缘融合) 处理重叠像素亮度 —— Pre-distort (预畸变) 矫正曲面投影变形

3.Architecture

3.1流水线架构

(1)CPU & GPU

Cpu: Control-heavy tasks; space for Control & Cache

Gpu: Data-heavy applications\ Lots of parallelism; space for ALU

(2)Multi-pipeline Synchronization

Software Synchronization（软件同步）

Frame Buffer Synchronization（帧缓冲区同步）

Video Synchronization（视频同步）

3.2分布式虚拟环境

(1) DVE (Distributed Virtual Environment)

(2) CVE (Collaborative Virtual Environment: A specific DVE that supports multiple users to complete tasks together in a virtual world

(3) 为什么 DVE 难

User hope instant response

Single server has to handle all clients、updates objects、distribute messages

Mer users lead to server overloaded

(4) Multi-server architectures

User partitioning: users assigned to different servers

Region partitioning: VE divided into fixed regions

Dynamic partitioning: regions adjusted at runtime

(5) 同步解决方案

Causal ordering (因果排序)：Events should be executed in the order they occur; only for discrete events

Roll-back (回滚机制)：

Object locking (对象锁定)

Dead Reckoning：Both transmitter and reviever use motion predictor for per motion-object{predictor predict position->set a error threshold value->only error<et value send update information:save network bandwidth}

(6) DVE 可连接:virtual museum\online games\remote education

4.Application

(1)VR for medical(virtual anatomy; the surgical process; DRE trainer; MIS)

Train medical students and doctors

Reduce cost

Support repeated practice and assignment

Allow errors in virtual patients not real one

（2）Manufacturing Applications

VADE (Virtual Assembly Design Environment)	：验证 CAD 设计装配

Jack：用于人机工程学分析的智能代理类人机器人 (humanoid agent)。

车身公差验证 (Car Body Tolerances)